# Quantum Communications Internship @ IRIDA

## Project Description

The main goal of this project is to investigate whether quantum properties, such as squeezing, can improve realistic communication channels based on coherent optical states.

We will study the phase-shift-keyed (PSK) binary communication protocol, where information is encoded and transmitted in coherent optical states and retrieved at the receiver using homodyne detection. Our aim is to determine whether the introduction of squeezing can improve the discrimination between the transmitted states and identify the optimal level of squeezing which minimizes the error probability. We will carry out this investigation both in ideal (noise-free) and realistic (noisy) channels. Our results will be directly compared with the ultimate quantum limits to discrimination, namely the Helstrom bound.

In this project we will follow closely the research paper “Squeezing-enhanced phase-shift-keyed binary communication in noisy channels” and try to reproduce their analytical results using the Strawberry Fields quantum photonic simulation framework. This implementation will provide an independent numerical validation of the reported results and enable systematic exploration of the protocol under different settings.

## Week 1 Overview (July 1-8)

- Defined Objectives/Plan for project

- Read reference paper & studied theoretical background

- Familiarized with Strawberry Fields (SF) software

- Prepared helper functions for state visualisation and measurement in SF

- Implemented coherent-state (CS) and displaced-squeezed-state (DSS) communication protocols for a noise-free channel

- Derived analytically the expected probability error

- Visualised the error probability for the aforementioned protocols and fitted it to the derived probability error

### Read reference paper & studied theoretical background

Reference paper: Gives the general idea on how the protocol works and provides the analytical expressions which we will try to reproduce using the SF software for all the different cases.

Theoretical background: Studied homodyne detection, squeezed/coherent states, and Helstrom bound to understand the underlying physics of the project.

### Familiarized with Strawberry Fields (SF) software

Read the SF software documentation and spent some time to familiarize with its conventions and learn about the potential applications.

### Prepared helper functions for state visualisation and measurement in SF

State Visualisation: Created functions that can visualise the Wigner function of a coherent state, squeezed-vacuum or displaced-squeezed vacuum according to the user’s inputs. The output is just a plot.

State Measurement: Created functions that can perform homodyne detection for a coherent state, squeezed-vacuum or displaced-squeezed vacuum according to the user’s inputs. The output is a single measurement following the statistics of the state.

### Implemented coherent-state (CS) and displaced-squeezed-state (DSS) communication protocols for a noise-free channel

Homodyne measurements are performed for the exchange of coherent states, which are either prepared as |α⟩ or |−α⟩ with equal probability. For each value of α, the measurement process is sampled many times, and the number of incorrectly classified states is recorded to estimate the error probability. This procedure is repeated over a range of values of α to investigate the behaviour of the error probability as a function of the average photon number.

The same protocol is then extended to displaced-squeezed states. In this case, the total energy is kept fixed while a fraction of the energy is allocated to squeezing. The error probability is studied as a function of the total photon number,

Ntot=|α|2+sinh2(r)

and the squeezing fraction,

β=sinh2(r)Ntot.

### Derived analytically the expected probability error

Although the reference paper outlines the initial steps of the derivation, it does not provide an explicit closed-form expression for the general error probability, but only its asymptotic behaviour in the limit N>>1 (Eqs. 16 & 17). Therefore, the next step was to derive the formula analytically and to check that the aforementioned limit agrees with the obtained formula:

Perr=12erfc(2Ntot(1−β)2Σ)

where

Σ=(Ntotβ+Ntotβ+1)−1

The derived expression was found to be in full agreement with the asymptotic results presented in the reference paper.

### Visualised the error probability for the aforementioned protocols and fitted it to the derived probability error

The produced data seem to follow well the derived error probability for both protocols and all parameters deviate less than 0.6σ from the expected value, validating our procedure so far.

## Next steps

- Compare the obtained results with those reported in the reference paper

- Determine the squeezing threshold for which the DSS protocol outperforms the CS protocol (βth)

- Determine the optimal squeezing fraction (βopt)

- Compare the error probability of both protocols with the corresponding Helstrom bound

## Week 2 Overview (July 8-15)

- Completed noise-free protocol:

  - Compared the obtained error probability plots with those in the reference paper

  - Derived analytical expressions for βth and βopt from the reference paper

  - Determined the squeezing threshold (βth) from the simulation

  - Determined the optimal squeezing fraction (βopt) from the simulation

  - Compared the error probability of coherent-state (CS) protocol and displaced-squeezed state (DSS) protocol with the corresponding Helstrom bound

  - Organized all files

- Started phase-diffusion protocol:

  - Explored theoretical results of the paper

  - Implemented phase-diffusion protocol in SF software and produced data for different σ

## Progress - Noise-free protocol

### Compared the obtained error probability plots with those in the reference paper

The simulated error probability surfaces were compared with those presented in the reference paper (Fig. 1) and were in excellent agreement over the investigated region.

### Derived analytical expressions for βth and βopt from the reference paper

Derived the squeezing fraction threshold (βth) and optimal squeezing fraction (βopt) (Eq. 8 from the paper) analytically. The squeezing threshold sets the upper limit of squeezing 0<β<βth for which the DSS protocol outperforms the CS protocol. The optimal squeezing fraction corresponds to the squeezing βopt that can achieve the lowest error probability. The expression for βth is obtained by setting PerrCS=PerrDSS and βopt is found by taking ∂PerrDSS∂β|N,βopt=0.

### Determined the squeezing threshold (βth) from the simulation

βth(N) was determined from the intersection of the fitted error probability surfaces of the CS and DSS protocols. For 0<β<βth the DSS protocol achieves a lower error probability than the CS protocol and is therefore advantageous. The intersection points were fitted against Eq. 8 of the paper, and the estimated parameters deviate less than 1.7σ from the theoretical predictions.

### Determined the optimal squeezing fraction (βopt) from the simulation

βopt(N) was determined by locating the minimum of the fitted DSS error probability surface for every average photon number N. The points were fitted against Eq. 8 of the paper, and the estimated parameters deviate less than 0.9σ from theoretical predictions.

### Compared the error probability of CS and DSS protocol with the corresponding Helstrom bound

Visualised Helstrom bound for CS and DSS protocol (Eq. 5 of paper), and compared it against the corresponding error probability (β=βopt was used in the DSS protocol). For the optimal fraction of squeezing, the DSS protocol outperfoms the CS protocol when it comes to reaching the ultimate (Helstrom) bound of discrimination. This indicates that allocating an optimal fraction of the available energy to squeezing, rather than entirely to displacement, improves the discrimination performance.

### Organized all files

Arriving at the end of the noise-free protocol signified a good point to organize the existent files and notebooks to make them readable and consistent (e.g. variable names). This establishes a solid workflow which can be followed in the next, more complex protocols such that they can be navigated smoothly.

## Progress - Phase-diffusion protocol

### Explored theoretical results of the paper

The next protocol that needs to be investigated is discrimination in the presence of phase noise. To implement this protocol, the theoretical framework of the paper was first studied (Eqs. 18-22). In this protocol the state becomes a statistical mixture of phase rotated CS or DSS. In particular, the state of the noise-free protocol |±a, r⟩ becomes Uϕ |±a, r⟩ = |±a e−iϕ, r e−i2ϕ⟩, where the phase ϕ is sampled from a Gaussian distribution centered at zero with standard deviation σ.

### Implemented phase-diffusion protocol in SF and produced data for different sigmas

To model phase diffusion, a large number of the aforementioned states are generated, homodyne measurement is performed, and the measurements are averaged over all sampled phases. This procedure is repeated for different values of σ to investigate how increasing phase noise affects the error probability.

## Week 3 Overview (July 15-22)

- Fitted theoretical predictions of the error probability Perr on data for different values of the noise parameter σ in phase-diffused CS/DSS protocol

- Compared the obtained error probability plots with those in the reference paper

- Determined the squeezing threshold (βth) from fitted data for different values of σ

- Determined (βth) numerically through theoretical expressions

- Determined the optimal squeezing fraction (βopt) from fitted data for different values of σ

### Fitted theoretical predictions of Perr on data for different values of the noise parameter σ

The theoretical predictions of Perr have a non analytical form in contrary to the noise-free model, due to integrating over the phase ϕ (Eq. 21 of the paper).

This expression is evaluated numerically using Gauss-Hermite quadrature for each (N, β) pair, and the estimated parameters are obtained through a fit which minimizes the sum of squared differences between the theoretical predictions and all simulated data points.

### Compared the obtained error probability plots with those in the reference paper

The simulated error probability surfaces were compared with those presented in the reference paper (Fig. 2) and showed excellent agreement over the investigated regions. As σ grows from zero, the corresponding surfaces of CS/DSS error probabilities are shifted upwards, and the minimum of the DSS surface is shifted towards lower β values. This observation indicates that for a fixed N, lower amount of squeezing is required to achieve the minimum error probability when σ is increased.

### Determined the squeezing threshold (βth) from fitted data for different values of σ

From the intersection of the fitted error probability surfaces of the CS and DSS protocols, βth is determined. The curve of βth(N) shifts to lower values for larger σ, indicating that noisier channels reduce the range of squeezing for which the DSS protocol can outperform the CS protocol.

### Found βth numerically through theoretical expressions

To theoretically validate the βth points taken from the intersection of the surfaces, the equation PerrDSS−PerrCS=0 was solved numerically using the estimated parameters found in the previous steps. The roots were found using Brent’s method provided from the SciPy library. The resulting threshold values were found to be in excellent agreement with those extracted directly from the intersection of the fitted surfaces.

### Determined the optimal squeezing fraction (βopt) from fitted data for different values of σ

From the DSS fitted error probability surface, the minimum value for each average photon number was extracted to determine βopt(N) for different values of σ. The results verify the aforementioned expectations.

## Next steps:

- Investigate Helstrom bound in CS/DSS phase diffusion protocol

- Compare the CS/DSS Helstrom bound to the corresponding error probability

- Investigate βth as a function of σ for constant values of N

- Complete phase diffusion protocol by organizing files and ensuring smooth workflow

## Week 4 Overview (July 22-29)

- Completed phase diffusion protocol:

  - Investigated βth as a function of σ for constant values of N

  - Calculated Helstrom bound for CS/DSS and compared it to the corresponding error probability

  - Changed the workflow after meeting with supervisor

  - Organized files according to the new workflow

- Started implementing protocol with thermal noise:

  - Studied theoretical results of the paper

  - Implemented the protocol in SF and produced MC data

  - Found βth/βopt from MC data

## Phase Diffusion Protocol

### Investigated βth as a function of σ for constant values of N

βth was plotted as a function of the noise parameter σ for fixed values of the average photon number N. The results show that for a fixed number of N, the threshold value βth is lower as the noise parameter σ increases. These results are in excellent agreement with those of the reference paper (Fig. 2).

### Calculated Helstrom bound for CS/DSS and compared it to the corresponding error probability

Calculated the Helstrom bound for CS/DSS as a function of the noise parameter σ with a fixed N. For the case of DSS, the optimal squeezing fraction was used. The calculation was carried out numerically by utilizing tools of the SF software. The results show that for low σ values, the DSS Helstrom bound is much lower than the corresponding CS values and for higher values of σ the two protocols seem to have a similar behaviour. The results are in excellent agreement with those of the reference paper (Fig. 3)

### Changed the workflow after meeting with supervisor:

- The second part of the project was initially intended to focus on implementing these protocols experimentally on Xanadu’s quantum photonic hardware. However, this was found to be infeasible because Xanadu’s cloud service, previously accessible through Strawberry Fields (SF), is no longer available. Consequently, the second part of the project was redirected towards a theoretical extension.

- Since SF draws measurements from the underlying probability distributions, it was decided to develop the Monte Carlo (MC) data-generation procedure analytically, without relying on the software implementation. This provides some insight into the mathematical background, and avoids treating the software as a black box. This method is also considerably faster, given that some processes can be parallelized, and MC data can be produced more quickly, even for large number of samples.

- The previous workflow (fitting the MC data to theoretical prediction and using the fitted surface to find βth/βopt), would be better if we intended to follow the experimental path. Since this project turned out to be solely a theoretical/computational investigation, it was decided that it is best to calculate statistical metrics to find the deviation of the MC data from the numerical solutions (since there is not always a closed form of the investigated quantity) for each step of the process.

### Organized files according to the new workflow

All protocols have been modified so that they follow the new workflow, as outlined in the previous point.

## Protocol with Thermal Noise

### Studied theoretical results of the paper

This protocol assumes an initially mixed thermal state rather than a pure vacuum state, on which the squeezing and displacement operator act upon. The purity of the seed state is defined as μ=Tr[ρ2], where ρ is the density matrix of the seed state.

This is the most general protocol considered so far, as it incorporates both the phase-diffusion parameter σ and the purity μ, allowing the study of displaced thermal states (DTS) and displaced squeezed thermal states (DSTS).

### Implemented the protocol in SF and produced MC data

To simulate the initial mixed state, a thermal state was created with average photon number Nth=(1−μ)/2μ. In the case of μ=1 (pure vacuum), the known results are retrieved. In this case, the average photon number is

N=|a|2+Nβ+1−μ2μ(1+2Nβ)

where |a|2>0, which imposes restrictions on the allowed values of β and μ for a fixed number of N. Since the investigation is now over the purity, σ and N are kept fixed, while μ takes values in the allowed range.

Data was produced from this protocol, and the resulting error probabilities were in excellent agreement with those of the reference paper (Fig. 4).

### Found βth/βopt from MC data

βth(μ) was determined from the MC data as the interpolated points satisfying Perr(DSTS)=Perr(DTS).

βopt(μ) was determined from teh MC data by locating the minimum of the DSTS error probability.

## Week 5 Overview (July 29 - August 5)

- Compared MC Data of βth/βopt to numerical predictions

- Calculated σth as a function of μ

- Calculated Helstrom bound for DTS/DSTS and compared it to the corresponding error probability

- Investigated effect of σth on Helstrom bound

- Made code more efficient for all protocols

- Started preparing for midterm presentation

### Compared MC Data of βth/βopt to numerical predictions

The equations PerrDTS−PerrDSTS=0 and d/dβ (PerrDSTS)=0 were solved numerically using Brent’s method provided from the SciPy library. The resulting threshold and optimal values βth/βopt were found to be in excellent agreement with those extracted from the MC data.

### Calculated σth as a function of μ

For the error probability Perr, there exists a threshold value of the noise parameter σth, beyond which there are no values of β for which DSTS achieves a lower error probability than DTS. If σ>σth, squeezing no longer provides an advantage over displacement alone.

To find σth from the MC data, a range of phase-diffusion values was scanned. For each fixed value of μ, the threshold was defined as the first value of σ for which βopt=0.

The threshold was also numerically found by solving

Perr(DSTS)​βopt(μ,σ)−Perr(DTS)=0

and it was found to be in excellent agreement with the MC data and results from the reference paper (Fig. 5).

### Calculated Helstrom bound for DTS/DSTS and compared it to the corresponding error probability

The Helstrom bound for DTS/DSTS was calculated numerically as a function of the purity μ for fixed values of the average photon number N and noise parameter σ. For the DSTS protocol, the optimal squeezing fraction was used.

### Investigated effect of σth on Helstrom bound

It was found that when σth is reached, the Helstrom bound becomes the same for DSTS and DTS, validating that squeezing offers no advantage.

### Made code more efficient for all protocols

Some time was spent optimizing procedures across all protocols, so that it is easier and faster to obtain the desired results.

### Started preparing for midterm presentation

On the 13th of August the midterm presentation will take place, thus until then, the desired results will be collected and organized in a presentation.

## Next steps:

- Finish preparations for presentation

- Decide on the extension of the project

## Week 6 Overview (August 5 - 12)

- Midterm presentation preparation

### Midterm presentation preparation

Prepared all necessary graphs and results and incorporated them into a structured power point presentation summarizing all work done up to this week. Additionally, revised all ideas and formulas studied so far and practiced the presentation to improve clarity and delivery.

## Week 7 Overview (August 12 - 19)

- Report writing

- Started developing the theoretical connection between photon loss and thermal noise

### Report writing

Started preparing the theoretical background and presenting the obtained results in the form of a research report.

### Started developing the theoretical connection between photon loss and thermal noise

Motivated by the midterm presentation feedback, I started investigating the theoretical connection between photon loss and thermal noise, with particular focus on Eqs. (23)–(25) of the reference paper.

## Next steps:

- Read the literature suggested by my supervisor and develop an understanding of the new concepts

- Identify and review relevant literature independently

- Continue report writing

## Week 8 Overview (August 19 - 26)

- Completed the theoretical investigation of the connection between photon loss and thermal noise in the initial vacuum state

- Completed the first draft of the report for the first part of the project

### Completed the theoretical investigation of the connection between photon loss and thermal noise in the initial vacuum state

Photon loss channel’s results can be obtained by relating the involved parameters (loss: η, squeezing: r) with the parameters of the channel with thermal noise in the initial vacuum state (purity: μ, squeezing: r). These relations can be obtained by equating the variances of the two conjugate quadratures, X0 and Xπ/2, for the two descriptions.

### Completed the first draft of the report for the first part of the project

The first draft of the report has been completed up to the results obtained in Week 8. It includes the Introduction, Methods, Results, and Conclusions sections. The report will be updated in the following weeks with the corresponding chapters for the second part of the project, while maintaining coherence and consistency between the two parts.

## Week 9 Overview (August 26 - September 3)

- Read literature suggested by supervisor and other papers relevant to the project

- Finalized the extension of the project with supervisor after proposing a new research question

- Started implementing coherent-state QKD protocol in noise-free channel

- Calculated mutual information IAB between Alice and Bob in coherent-state noise-free channel

### Read literature suggested by supervisor and other papers relevant to the project

Many papers were studied during this period to make an informed choice on the project’s extension. The topics considered included extending the protocol from binary phase-shift keying (BPSK) to higher-order phase-shift keying schemes, incorporating entanglement into the existing protocol, investigating the role of squeezing in continuous-variable (CV) quantum key distribution (QKD) protocols, exploring non-Gaussianity as a resource, etc.

### Finalized the extension of the project with supervisor after proposing a new research question.

From the different research directions considered above, the investigation of the potential advantage of squeezing in QKD protocols was particularly interesting for me. Since the previous part of the project focused on investigating squeezing in the BPSK protocol, it was considered valuable to extend the investigation to CV QKD. I proposed my idea to the team and, following discussion with my supervisor, it was agreed to proceed with this research direction. The main reference that inspired this idea is Squeezed-state quantum key distribution upon imperfect reconciliation by Vladyslav C. Usenko and Radim Filip 2011, New Journal of Physics 13, 113007.

The idea behind the protocol is:
Alice prepares coherent states whose phase and amplitude quadratures are modulated according to Gaussian distributions. Bob randomly chooses one of the two quadratures and measures it using homodyne detection. He then publicly announces which quadrature was measured, allowing Alice to retain only the corresponding modulation values for the key generation process. Alice and Bob subsequently perform either direct reconciliation (DR) or reverse reconciliation (RR). In DR, Bob performs error correction using Alice’s data as the reference, whereas in RR, Alice performs error correction using Bob’s data as the reference. The reconciliation efficiency, denoted by b, quantifies how close the reconciliation procedure is to the ideal case, with b=1 corresponding to perfect reconciliation and b<1 to imperfect reconciliation. Assuming that an eavesdropper, Eve, can access the channel through a beamsplitter attack, the maximum amount of information available to Eve can be quantified using the Holevo bound, χE. The objective is then to calculate the lower bound of the secure key rate (SKR), which quantifies the rate at which Alice and Bob can generate a secure key while limiting Eve’s information about it. The main goal of this investigation is to compare the SKR in the presence and absence of squeezing and determine whether squeezing can provide an advantage as a resource in CV-QKD protocols.

### Started implementing coherent-state QKD protocol in noise-free channel

The protocol has been implemented using Strawberry Fields (SF). Alice samples from a Gaussian distribution two values (ap,ax) and uses them to modulate the phase and amplitude quadratures of the coherent state, respectively. Bob measures randomly one of the two quadratures using homodyne detection and Alice retains only the corresponding modulation values for the key generation.

### Calculated mutual information IAB between Alice and Bob in coherent-state noise-free channel

From the retained values of Alice and the measurements of Bob, we can calculate the quantity of mutual information (IAB), which quantifies the amount of information that Bob’s measurement provides about Alice’s modulation values and therefore characterizes the correlation between their data. This quantity is very useful as it can be used to obtain the lower bound of the SKR which is equal to:

K=IAB*b−χE

## Next steps:

- Calculate the Holevo bound for RR and DR in noise-free coherent-state channel

- Calculate the mutual information between Alice and Bob and the Holevo bound for RR and DR in the noise-free displaced-squeezed-state channel

## Week 10 Overview (September 3 - 9)

- CS QKD protocol

  - Calculated the Holevo bound for RR and DR

  - Calculated the lower bound of the secure key rate (SKR)

  - Compared the SKR results with the literature

- DSS QKD protocol

  - Implemented the protocol

  - Calculated the mutual information IAB between Alice and Bob

## CS QKD protocol

### Calculated the Holevo bound for RR and DR

The Holevo bound depends on the von Neumann entropy, which can be expressed in terms of the bosonic entropy function

G(ν)=ν+12log2ν+12−ν−12log2ν−12,

where ν is the symplectic eigenvalue of the corresponding covariance matrix. The covariance matrix of Bob and Eve is obtained from SF and accounts for the quantum quadrature variances. The classical modulation variance Vα is subsequently added to the covariance matrix to account for the Gaussian modulation.

The Holevo bound was then calculated separately for DR and RR using the corresponding conditional covariance matrices.

### Calculated the SKR of DR/RR

The lower bound of the SKR for DR and RR was calculated using

K=bIAB−χE,

where b is the reconciliation efficiency, IAB is the mutual information between Alice and Bob, and χE is the corresponding Holevo bound. Specifically, χE=χAE for DR and χE=χBE for RR.

A negative value of the lower bound of the SKR indicates that no positive SKR can be guaranteed for the considered channel parameters and the channel is not secure. As expected, the SKR increases with increasing channel transmission η and reconciliation efficiency β.

### Compared results of SKR to literature for DR/RR

The calculated results were compared with those reported in the reference paper (Fig. 2) and showed excellent agreement. One important observation is that the SKR exhibits a maximum as a function of the modulation variance, indicating that the modulation variance should be optimized to maximize the SKR.

Additionally, RR appears to tolerate lower channel transmissions than DR, making it more robust against channel loss.

## DSS QKD protocol

### Implemented the protocol

Alice randomly chooses either the x or p quadrature to squeeze and then samples the displacement of each quadrature from a Gaussian distribution with variance Vα. Bob randomly chooses one of the two quadratures and measures it using homodyne detection. Alice and Bob then publicly announce which quadrature was squeezed and which quadrature was measured, and retain only the values for which the two choices coincide.

To simplify the simulated protocol and reduce the number of discarded runs, we fix the squeezing choice to the amplitude quadrature, as in the reference paper, without loss of generality. This is possible because the modulation values for the two quadratures are sampled from the same Gaussian distribution, making the protocol symmetric with respect to the choice of quadrature.

### Calculated the mutual information IAB between Alice and Bob

Using the retained values of Alice’s modulation and Bob’s homodyne measurements, we calculate the mutual information IAB in the presence of squeezing. The results show that squeezing enhances the mutual information, as the quantum fluctuations in the squeezed quadrature are reduced, making Alice’s modulation more distinguishable at Bob’s detector.

## Week 11 Overview (September 9 - 16)

- DSS QKD protocol

  - Calculated the Holevo bound for RR and DR in the DSS channel

  - Calculated the lower bound of the SKR for the DSS channel and compared the results to literature

- CS & DSS

  - Found the threshold value of reconciliation efficiency bth for DR/RR

  - Compared the CS and DSS results

  - Developed code that produces MC data without SF

## DSS QKD protocol

### Calculated the Holevo bound for RR and DR in the DSS channel

The Holevo bound for the DSS protocol was calculated for both DR and RR. The same approach used for the CS protocol was applied, with the covariance matrices constructed according to the DSS protocol.

### Calculated the SKR for the DSS channel and compared results to literature

The SKR was evaluated for different channel transmissions, modulation variances, and squeezing levels. The results were compared with the results reported in the reference paper and showed excellent agreement.

In particular, squeezing was found to be a useful resource since it seems to increase the SKR in some cases.

## CS & DSS

### Found the threshold value of reconciliation efficiency bth for DR/RR

The threshold reconciliation efficiency was calculated to determine the minimum value of b required to obtain a positive lower bound of the SKR for each value of η.

The threshold values were calculated separately for DR and RR and compared between the CS and DSS protocols. This comparison identifies the regions where squeezing improves the SKR, is necessary to obtain a positive SKR, or provides no benefit.

### Developed code that produces MC data without SF

Since SF was time-consuming for generating MC data, another protocol without SF was developed that is faster. SF is a useful tool for initially understanding the underlying processes, as it provides a more intuitive way of implementing the protocol. However, once the underlying concepts are understood, other approaches can be used to generate the MC data more efficiently.

## Next steps:

- Organize/Optimize codes and prepare results in desired format

- Report writing for the second part of the project

- Final presentation for the second part of the project